NOTE: This document was downloaded from the web, converted to document format and formatting removed. NEHSTC members wishing to edit/comment, etc should copy to local drive use reviewing tools, make edits and send back with initials at end (…doc_JT.doc). Send to jim.turenne@gmail.com

Edits will be reviewed by the committee and voted on for adoption.

Field Indicators for Identifying Hydric Soils in New England 
Version 4 April 20??

DRAFT | DRAFT | DRAFT | DRAFT | DRAFT | DRAFT |

The New England Hydric Soils Technical Committee

(NEHSTC) consists of the following members:

Peter Fletcher, Retired from USDA - Natural Resources Conservation Service

James Gove, Gove Environmental Services

Joseph Homer, USDA - Natural Resources Conservation Service – New Hampshire

Ruth Ladd (Chair), US Army Corps of Engineers

Harvey Luce, University of Connecticut

Joseph Noel, Maine Association of Professional Soil Scientists

Thomas Peragallo (Co-Chair), Peragallo Associates

Jeff Peterson, Vanasse Hangen Brustlin

Sidney Pilgrim, University of New Hampshire

David Rocque, Maine Dept. of Agriculture, Food and Rural Resources

Michael Sheehan, US Army Corps of Engineers

Mark Stolt (Co Chair), University of Rhode Island

James Turenne, USDA – Natural Resources Conservation Service – Rhode Island

Peter Veneman, University of Massachusetts, Amherst

Thomas Villars (Co Chair), USDA – Natural Resources Conservation Service - Vermont

Paul Minkin, US Army Corps of Engineers

Rebecca Chalmers, VTANR

Caroline Alves, USDA-NRCS

Steve Sprecher, USDA-NRCS

Karen Dudley, USDA-NRCS

Greg Granger, USDA-NRCS

Shawn Finn, USDA-NRCS

Al Averill, USDA-NRCS

Donald Parizek, USDA-NRCS

Gino Infascelli, NHDES

Erica Sachs, U.S. EPA

Kerry Strout (Honarary) NEIWPCC

Christine Delorier, US Army Corps of Engineers

Michael Whited (Honarary), USDA – Natural Resources Conservation Service

This manual should be cited as follows:

New England Hydric Soils Technical Committee. 20??. 4th ed., Field Indicators for Identifying Hydric Soils in New England. New England Interstate Water Pollution Control Commission, Lowell, MA.

In the mid-1980s, staff from the New England Division of the Army Corps of Engineers, with technical assistance provided by the United States Department of Agriculture (USDA) Natural Resources Conservation Service (then the Soil Conservation Service), began developing New England-wide soil drainage class criteria to make field identification of hydric soils easier and more standardized. Several years of exhaustive field testing were conducted, involving a myriad of consulting firms, academic experts, professional societies, and federal, state, and local agencies.

Over the years, the soil drainage class criteria were revised many times as the knowledge base and experience grew. To gather data and obtain opinions, detailed questionnaires were circulated to soil scientists, wetland ecologists, and others in regulatory agencies, private consulting firms, and universities. Despite significant improvements, the criteria for the drainage classes were still confusing at times, and some of the region’s soil scientists had reservations about using them.

In 1992, in response to efforts at the federal level to develop field indicators of hydric soils on a nationwide basis, a group of federal, state, private and academic experts in wetland and soil sciences (many of the same individuals from the earlier effort) convened to develop regional field indicators for identifying hydric soils in New England. Numerous drafts were produced, circulated for review, revised, and recirculated. The proposed field indicators were tested throughout New England by a variety of soil scientists and others in the wetlands profession. This manual is the culmination of those efforts.

Following the release of Version 1, the New England Hydric Soils Technical Committee and representatives from the National Technical Committee for Hydric Soils (NTCHS) had several joint meetings and field trips to develop consistency between the New England and National Indicators. As a result of that cooperative effort, Version 2 of Field Indicators for Identifying Hydric Soils in New England included revisions in terminology to more nearly parallel the national indicators and additional clarification of indicators which increased both accuracy and consistency. Likewise, the NTCHS has made, and continues to make, revisions to the National Indicators that better reflect hydric soil morphologies indicative of the New England Region. Version 3 of the Field Indicators for Identifying Hydric Soils in New England continues this process.

Add continuation info v 3 to v 4.

The purpose of this field indicators guide is to make it easier for field personnel to identify and document hydric soils and to provide consistent and reliable evidence as to whether a certain soil meets the definition of a hydric soil. To correctly interpret and apply this guide, however, users must have practical experience and a working knowledge of soils. When properly applied, this field guide yields results that are consistent with the identification of hydric soils as per the 1987 Army Corps of Engineers Wetland Delineation Manual.

While this field guide is the culmination of more than fifteen years of work, it is by no means a final product. Practical application, experience and continuing academic research provide sufficient reason to continue to periodically revise and update the field guide. Much of the supporting documentation for those revisions will come from you, the user.

Ahydric soil is a soil that formed under conditions of saturation, flooding, or ponding long enough during the growing season to develop anaerobic conditions in the upper part (Federal Register, July 13, 1994). As a result of soil saturation and reducing conditions, hydric soils undergo chemical reactions and physical processes which differ from those found in upland soils. The field indicators of hydric soils contained within this guide are observable soil morphologies that have developed as a result of those chemical reactions and physical processes. Field indicators are also a historic record of cycles

Terms or phrases in bold italics are defined in the Glossary of Terms

of soil saturation and reduction that have been observed and documented by field soil scientists and validated through testing.

For many hydric soils, saturation and reduction in the upper part of the soil are seasonal and not continuous throughout the growing season. When the soil morphologies observed at a site meet the criteria for one of the New England field indicators, this specifies a hydric soil. Users are cautioned that when working in areas where the hydrology has been altered, the presence or absence of hydric soil indicators may not reflect the current hydrologic conditions.

This key is specifically designed to address the hydric soil conditions common to New England and may not be applicable to other regions of the United States. Soils identified as hydric soils using this key meet the definition of a hydric soil.

This key begins with soil conditions associated with the wettest soils—those with extended periods of saturation in the upper part and/or inundation. As the user progresses through this key, the soils have shorter periods of saturation in the upper part and the evidence of wetness may not be as clearly expressed in the soil morphology. To avoid misinterpretation when evaluating these soils, other than histosols, this key requires the user to identify a combination of soil morphologies observed within specified depths.

Because of the unique soil morphology associated with spodic horizons, these are addressed separately in this key. In addition, some indicators are grouped by soil texture. To use this key properly, a user must start with Hydric Soil Indicator I and progress through the key systematically. If the soil conditions observed in the field match one of the hydric soil indicators, that soil is considered to be hydric.

This key was designed to be an easy-to-use field guide that would apply throughout the New England Region. With the myriad of soil conditions found in New England, it became evident early on that one key could not address all hydric soil conditions without becoming very complex and difficult to use. Hence, this key addresses soil conditions most commonly associated with hydric soils in New England. It also recognizes that there are “problem soils” for which this key may not work properly. Indicator XIII has been greatly expanded to help the user understand some of the types of problem soils that may be encountered in New England. A separate supplement to this manual is planned, which will reference regional maps to be located on the NEIWPCC website showing the approximate extent of problem soil areas and a list of reliable field indicators for them.

When documenting hydric soils in the field, it is recommended that soil profiles be observed in test holes dug to a depth of two feet or more using a tile spade. The soil conditions observed in each test hole should then be described and documented using standards and terminology developed by the USDA. Depth measurements should always be made from the soil surface, but it is important to note that some indicators refer to the distance below the top of the mineral soil material or some other reference point as noted in the Indicators. Soil conditions and features that need to be observed and documented in order to use this key effectively include:

- Soil horizons and layers – list the horizon designation plus thickness and depth for each.

- Soil texture – note the textural class.

- Soil color – list the Munsell Notation for matrix color and other features as described under ‘redoximorphic features’ below.

- Redoximorphic features:

– Redoximorphic concentrations/depletions - Describe each type and document their color, abundance, size, contrast, and location (e.g., ped interiors). Field identification of redoximorphic features is most reliable when their contrast is distinct or prominent, and their abundance is 2 percent or more. When features are faint and/or less than 2 percent, extra care and professional

judgment should be exercised as to whether or not they are an indication of a hydric soil.

– Other redoximorphic features – Depleted or gleyed matrix, nodules, pore linings, etc.

- Soil structure – describe type (shape), size, and grade.

- Carbon-based features – histic epipedon, histosols, thick mucky surface layers, and stripped matrix.

- Other soil features may be required depending upon the situation (e.g., depth of rooting, soil consistence).

This manual includes a series of diagrams which illustrate each hydric soil indicator. The scales on either side of the diagram show the reference point of measurement. Soil textures are depicted using contrasting patterns. Notes highlight important characteristics and morphologies. Some features are difficult to show graphically and if there is a perceived difference in interpretation between the diagrams and narrative key, the narrative key takes precedence.

User Notes are shown on the page opposite the diagram and below the indicator that they refer to. Their purpose is to provide additional information which may help in identifying an indicator in the field, or to alert users to possible exceptions and/or situations where professional judgment may be needed. User Notes are identified by the symbol shown on the left.

In addition to User Notes, soil features designated as Field Testing Only are soil morphologies which may be helpful when identifying field indicators, but users are cautioned that these features have not had the field testing needed to substantiate their reliability. People who have observed these soil features in the field should document their findings and forward them to the NEIWPCC Wetlands Group. Your active participation is greatly appreciated and will improve and strengthen this manual. Field Testing Only criteria are depicted by the symbol shown on the left. Standards for defining a functionally hydric soil are maintained by the National Technical Committee at the following website:

http://soils.usda.gov/use/hydric/criteria.html.

This manual also includes a Glossary of Terms. The terms included in the Glossary are in bold and italicized throughout the narrative text. Some definitions have been simplified to facilitate a field interpretation. Users who want complete definitions should refer to the references cited.

Please note that this manual does not address all hydric soil conditions found in New England, and professional judgment by a qualified individual is needed in unusual or problem soil areas.

Included with this version of the Field Indicators is a separate document entitled Supplement to the Field Indicators of Hydric Soils in New England. The supplement explains some of the nuances of assessing soils; it is intended as an aide for less experienced individuals. The supplement also includes guidance to assist in the evaluation of problem soils.

References

Federal Register. February 24, 1995. Hydric Soils of the United States. Washington, DC (current Hydric Soil Criteria).

Federal Register. July 13, 1994. Changes in Hydric Soils of the United States. Washington, DC (current Hydric Soil Definition).

Glossary of Soil Science Terms. 1997. Soil Science Society of America, Madison, WI.

Tiner, R.W., Jr., and P.L.M. Veneman. 1995. Hydric Soils of New England. University of Massachusetts Extension, Revised Bulletin C-183R, Amherst, MA.

Department of Agriculture, Natural Resources Conservation Service. 2002. Field Indicators of Hydric Soils in the United States, Version 5.0. G.W. Hurt, P.M. Whited, and R.F. Pringle (eds.). USDA-NRCS in cooperation with the National Technical Committee for Hydric Soils, Fort Worth, TX.

Department of Agriculture, Natural Resources Conservation Service, 2002. National Soil Survey Handbook, title 430-VI. [Online] Available: http://soils.usda.gov/technical/handbook.

Department of Agriculture, Soil Survey Division Staff. 1993. Soil Survey Manual. Handbook No. 18. U.S. Government Printing Office. Washington, DC.

Department of Agriculture, Soil Survey Division Staff. 1999. Soil Taxonomy, Second Edition. Handbook No. 436. U.S. Government Printing Office, Washington, DC.

Vepraskas, M.J. 1996. Redoximorphic Features for Identifying Aquic Conditions. Technical Bulletin 301. North Carolina Agricultural Research Service, North Carolina State University, Raleigh, NC.

Version 4 – April 20??

CAUTION: The field indicators in this book should not be considered all-inclusive. Although an attempt has been made to list the most relevant indicators, there are cases (see Problem Soils discussion on p. 54) where hydric soils exist that do not meet any of the currently listed indicators. Therefore, not meeting one of the indicators does not preclude a soil from being determined as hydric. In addition, there are rare occasions when a soil has one or more relevant field indicators but is not a hydric soil. Field indicators will continue to be revised as problem areas and exceptions to these keys become known or additional interpretation tools are developed. When soil morphology seems inconsistent with the landscape, vegetation, or observable hydrology, the assistance of an experienced soil or wetland scientist may be needed to determine whether the soil is hydric.

The following conditions and soil morphologies are recognized as field indicators of soils that have formed under conditions of saturation, flooding, or ponding long enough during the growing season to develop anaerobic conditions in the upper part:

I. Ponded or Flooded Soils. Any soil that is frequently ponded or flooded for a long or very long duration during the growing season; or

II. Tidal Soils. Soils that are flooded daily by tides and/or have sulfidic materials within 12 inches of the soil surface; or

III. Histosols. Soils having organic soil materials at or near the soil surface that are greater than 16 inches thick (i.e., classify as Histosols)1; or

1. Folists and soils with folistic epipedons are excluded from Hydric Soil Indicator III.

IV. Histic Epipedons. Soils having a layer of organic soil materials at or near the soil surface that is 8 to 16 inches thick (i.e., histic epipedon); and directly underlying the O horizon or, if present, the A horizon is a horizon with redoximorphic features;2or

V. Mineral Histic. Any mineral soil having a layer 4 to 8 inches thick of slightly to well-decomposed organic soil material and/or a mucky A or Ap horizon, and is directly underlain by a horizon with a depleted or gleyed matrix;2 or

VI. Depleted or Gleyed Matrix. Within 10 inches of the top of the mineral soil material and directly underlying an A or Ap horizon (or, if they are not present, an O horizon), is a horizon with a depleted or gleyed matrix (for soils with moderate to strong structure, the matrix color is recorded for ped interiors); or

VII. Depleted Below Dark Surface. Beginning within 20 inches of the top of the mineral soil material and directly underlying a thick or very thick, dark A or Ap horizon is a horizon with a depleted or gleyed matrix that is 4 inches or more thick (for soils with moderate to strong structure, the matrix color is recorded for ped interiors); or

VIII. Dark Mineral Soils. Soils with a matrix chroma of 2 or less that extends to a depth of 20 inches below the top of the mineral soil material, and that have a dark A or Ap horizon (with or without an O horizon) that is directly underlain by a horizon with a matrix value of less than 4, and within 12 inches of the top of the mineral soil material or directly underlying an A or Ap horizon, whichever is shallower, 2 percent or more redoximorphic features that extend to either:

2. Soils with folistic epipedons are excluded from Hydric Soil Indicators IV and V. User Note: refer to page opposite the diagram depicting this indicator.

A. a depth of 20 inches below the top of the mineral soil material; or

B. a depleted or gleyed matrix, whichever is shallower; or

IX. Spodosols. Mineral soils having a spodic horizon and one of the following morphologies:

A. within 6 inches of the top of the mineral soil material have an E horizon (eluvial horizon) with 2 percent or more redoximorphic features and/or have a stripped matrix that is directly underlain by a spodic horizon with either:

- a Bh, Bhs, or Bs horizon with 2 percent or more redoximorphic features; or

- a Bh and/or Bhs horizon that is directly underlain, but within 20 inches of the top of the mineral soil material, by a horizon with 2 percent or more redoximorphic features; or

B. beginning within 10 inches of the top of the mineral soil material and directly underlying a dark A or Ap horizon and/or a shallow E horizon (or if neither is present, an O horizon), there is a Bh and/or Bhs horizon3 that is greater than 2 inches thick that is directly underlain, but within 20 inches of the top of the mineral soil material, by a horizon with 2 percent or more redoximorphic features; or

3. This morphology can develop as a result of wetness, or in the cryic temperature regime, as a result of climate. See Section XIII for problem soil areas due to cold temperatures.

C. beginning within 10 inches of the top of the mineral soil material and directly underlying a dark A or Ap horizon, is one of the following:

- an E horizon with 2 percent or more redoximorphic features and/or a stripped matrix directly underlain by a Bh, Bhs, or Bs horizon with 2 percent or more redoximorphic features;

- a Bh and/or Bhs horizon that is directly underlain, but within 20 inches of top of the mineral soil material, by a horizon with 2 percent or more redoximorphic features; or

- a Bs horizon with 2 percent or more redoximorphic features; or

X. Sandy With Redox. Soils that do not have a spodic horizon and:

A. have a layer 4 to 8 inches thick of slightly to well-decomposed organic soil material and/or a mucky A or Ap horizon; and are directly underlain by a horizon with a loamy fine sand or coarser texture with 2 percent or more redoximorphic features; or

B. beginning within 10 inches of the top of the mineral soil material and directly underlying a dark A or Ap horizon (with or without an O horizon) is a horizon with a loamy fine sand or coarser texture with a matrix color due to wetness of chroma 3 or less, value 4 or more, with 2 percent or more redoximorphic features; or

C. beginning within 15 inches of the top of the mineral soil material and directly underlying a thick, very dark A or Ap horizon there is a horizon with a loamy fine sand or coarser texture with a matrix color due to wetness of chroma 3 or less, value 4 or more, with 2 percent or more redoximorphic features;

XI. Any Texture. Soils that do not have a spodic horizon and

A. beginning within 10 inches of the top of the mineral soil material and directly underlying a dark A or Ap horizon (with or without an O horizon) is a horizon with 5 percent or more redox depletions, and within 20 inches of the top of the mineral soil material there is a horizon with a depleted or gleyed matrix (for soils with moderate to strong structure, the matrix color and percent redox depletions are recorded for ped interiors); or

B. beginning within 15 inches of the top of the mineral soil material and directly underlying a thick, very dark Ap horizon is a horizon with 5 percent or more redox depletions and within 20 inches of the top of the mineral soil material there is a horizon with a depleted or gleyed matrix (for soils with moderate to strong structure, the matrix color and percent redox depletions are recorded for ped interiors); or

XII. Any Texture. Soils that have a very dark A or Ap horizon less than 10 inches thick (with or without an O horizon) that are directly underlain by a horizon with a matrix color due to wetness of chroma 3 or less, with 10 percent or more redoximorphic features; and within 6 inches of the top of the mineral soil material have 2 percent or more redoximorphic features; and within 18 inches of the top of the mineral soil material have 2 percent or more redox depletios; or

XIII. “Problem” Soil Areas. This category addresses a variety of conditions for which definitive indicators have yet to be developed. The term “problem area wetlands” was coined in the 1987 U.S. Army Corps of Engineers Wetland Delineation Manual and is used to describe environments where one or more of the wetland parameters are missing or difficult to interpret. The final evaluation of hydric status of these soils is left to professional judgment

In Version 2 of this manual, the need was recognized to “identify general locations of these unique soils and to develop reliable field indicators for identifying hydric soils within them.” This need is also recognized by the U.S. Army Corps of Engineers (Wakeley, 2002). Information from the USDA-STATSGO maps available for the New England region provide a reasonable picture of where many of the problem soils may be encountered. Regional soil maps identify the general location of problem soil areas at a scale of 1:250,000. They are available at the NEIWPCC website: www.neiwpcc.org/hydricsoils.htm

In 2002 the New England Hydric Soils Technical Committee (NEHSTC) established sub-committees to begin research and development of keys for identifying hydric soils in the problem soil areas. Soil scientists with specific experience and expertise in respective problem areas lead the sub-committees. The subcommittees are investigating: 1) Disturbed soils 2) Folists and Soils with Folistic Epipedons 3) Soils formed in Dark Parent Material and 4) Soils formed in Red Parent Material. All subcommittees have been assigned three principal tasks: summarizing the principal difficulties in making hydric soil determinations in these areas, developing appropriate research and monitoring of “bench mark” sites, and producing a key that will be useable in the professional wetland science community. A detailed description of these areas is found in the section on Problem Soil Areas starting on page 54 of this manual.

Glossary

A Horizon – Mineral soil horizons which have formed at the surface or below an O horizon which show an accumulation of humified organic matter intimately mixed with the mineral fraction. An A horizon has a morphology distinct from the C layer, although the mineral fraction is unaltered or slightly altered by weathering. Such a horizon is designated A because it is at the surface; however, recent alluvial or eolian deposits that retain rock structure are not considered to be an A horizon unless cultivated. See Soil Survey Manual (1993) for complete definition.

A or Ap Horizon, Dark5– A or Ap horizon that has moist colors with chromas 2 or less and values 3 or less.

A or Ap Horizon, Very Dark –A or Ap horizon that has moist colors with chromas 2 or less and values less than 3.

Abundance, Redoximorphic Features – The percent of the surface area covered by redoximorphic features. Refer to Chart for Estimating Percent Redoximorphic Features, located in Appendix 1.

Albic Horizon – An eluvial horizon 0.4 inches (1 cm) or more thick that has 85% or more (by volume) albic materials. See Keys to Soil Taxonomy (2003).

Albic Materials – Mineral soil materials from which clay and free iron oxides have been removed or in which the oxides have been segregated to the extent that the color of the horizon is determined primarily by the color of the primary sand and silt particles rather than by the coatings on these particles. See Keys to Soil Taxonomy (2003).

Anaerobic Conditions – A condition when molecular oxygen is essentially absent from the soil.

Ap Horizon – An A horizon with properties resulting from cultivation, pasturing, or similar kinds of disturbance.

5. Definition has been modified to better reflect the conditions common to New England and may vary from the National definition.

Aquic Conditions – Soils with aquic conditions are those that currently undergo continuous or periodic saturation and reduction. The presence of these conditions is indicated by redoximorphic features, except in Histosols, and can be verified—except in artificially drained soils—by measuring saturation and reduction. See Keys to Soil Taxonomy (2003) for complete discussion.

The three elements of aquic conditions are:

- Saturation – Characterized by zero or positive pressure in the soil-water and can generally be determined by observing free water in an unlined auger hole.

- Reduction – The degree of reduction in a soil can be characterized by the direct measure of redox potential. The a,a’dipyridyl test (Childs, C.W., 1981) may be used in the field to determine the presence of reduced iron (ferrous, Fe+2) ions.

- Redoximorphic features – Associated with wetness; result from the reduction and often oxidation of iron and manganese compounds in the soil after saturation with water and desaturation, respectively.

B Horizon – Horizon that forms below an A, E, or O horizon that shows evidence of soil formation, i.e. illuviation, development of soil color, formation of soil structure, etc. See Soil Survey Manual (1993) for complete definition.

Bh and Bhs Horizons – B horizons with an accumulation of illuvial, amorphous, dispersible organic matter (sesquioxide complexes). The sesquioxide component coats sand and silt particles. The symbol “h” is used in combination with “s” as Bhs if the amount of sesquioxide component is significant but value and chroma of the horizon are 3 or less. These horizon designations are often used to identify and describe spodic horizons. See Soil Survey Manual (1993) for complete definition.

Biological Zero – the soil temperature, at a depth of 50 cm (19.7”), below which the growth and function of locally adapted plants are negligible. (Reference: NRCS Hydric Soil Criteria as posted on http://soils.usda.gov/use/hydric/criteria.html.) For the purposes of this manual, a temperature of 5°C (41°F) is used.

Bs Horizon – B horizon with an accumulation of illuvial, amorphous, dispersible organic matter – sesquioxide complexes if both the organic matter and sesquioxide components are significant, and if either the moist color value or chroma of the horizon is more than 3. This horizon designation is often used to identify and describe spodic horizons. See Soil Survey Manual (1993) for complete definition.

Common, Redoximorphic Features – Redoximorphic features comprising from 2 to 20 percent of the observed surface area.

Concretion (Fe/Mn) – A cemented concentration of a chemical compound of iron and manganese that can be removed from the soil intact and that has a crude internal symmetry organized around a point, a line, or a plane.

Contrast, Redoximorphic Features – The contrast in color between the redoximorphic feature and the matrix color. See faint, distinct, and prominent redoximorphic features. Refer to both the Chart and the Table for Estimating Contrast of Redoximorphic Features, located in Appendices 3 and 4.

Crushed and Smoothed Color – A soil color recorded for a moist soil sample that has been broken up, and then lightly compressed and smoothed with one’s thumb. This is used when documenting colors of spodic horizons.

Cryic Temperature Regime – A soil temperature regime that has mean annual soil temperatures of less than 8 oC. Mean summer soil temperature for saturated mineral soils with O horizons is lower than 6 oC. See Keys to Soil Taxonomy (2003) for complete definition.

Dark A or Ap Horizon6 – Refer to A or Ap Horizon, Dark.

Depleted Matrix6– A depleted matrix makes up more than 50% of the horizon or subhorizon from which iron has been removed or transformed by processes of reduction and often translocation to create colors of low chroma and high value. A and E horizons may have low chromas and high values and may therefore be mistaken

6. Definition has been modified to better reflect the conditions common to New England and may vary from the National definition.

for a depleted matrix; however, they are excluded from the definition, unless, in the case of an E horizon, it has 2 percent or more redoximorphic concentrations. In some places the depleted matrix may change color upon exposure to air (reduced matrix); this phenomenon is included in the concept of depleted matrix.

The following combinations of value and chroma identify a

depleted matrix:

- Matrix value 4 or more and chroma 1 with or without other redoximorphic features;or

- Matrix value 6 or more and chroma 1 or 2 with or without other redoximorphic features; or

- Matrix value 4 or 5 and chroma 2 or less with 2 percent or more redoximorphic features. When features are faint, extra care and professional judgment should be exercised as to whether or not they are an indication of a hydric soil.

For matrix color matching those on the color pages labeled

“Gley,” refer to gleyed matrix definition.

Directly Underlain, Horizon – A soil horizon which lies immediately beneath, and is in direct contact with, another soil horizon. When such a horizon is required to have specific soil morphological features, those features must be present throughout that horizon.

Distinct, Redoximorphic Feature – A term used to describe the contrast in color between a redoximorphic feature and the matrix color. Refer to both the chart and tabular key for estimating contrast of redoximorphic features, located in Appendices 4 and 5.

E Horizon – A mineral soil horizon in which the main feature is loss of organic matter, silicate clay, iron, or aluminum, or some combination of these, leaving a concentration of sand and silt particles. See Soil Survey Manual (1993) for complete definition. This horizon designation is often used to identify and describe albic horizons.

Faint, Redoximorphic Feature – A term used to describe the contrast in color between a redoximorphic feature and the matrix

color. Refer to both the chart and tabular key for estimating contrast of redoximorphic features, located in Appendices 4 and 5.

Fe/Mn Concretion – See Concretion.

Fe/Mn Nodule – See Nodule.

Few, Redoximorphic Features – Redoximorphic features comprising less than 2 percent of the observed surface area.

Fibers (Organic) – Pieces of plant tissue in organic soil material, excluding live roots. The amount of fibers after rubbing (rubbed fiber content) is used to differentiate fibric, hemic and sapric organic soil materials. See Keys to Soil Taxonomy (2003) for complete description.

Fibric Soil Material – Organic soil material with a fiber content after rubbing of two fifths or more recognizable fibers of undecomposed plant remains, excluding live roots. The soil horizon notation for fibric soil material is Oi. See Soil Survey Manual (1993) for complete definition.

Finer Than Loamy Fine Sand – Includes USDA Textural classes: very fine sand, loamy very fine sand, all sandy loam textures, loam, silt loam, silt, sandy clay loam, silty clay loam, clay loam, sandy clay, silty clay, and clay.

Flooded – A condition in which the ground surface is temporarily covered with flowing water from sources such as streams overflowing their banks, runoff from adjacent or surrounding slopes, inflow from high tides, or any combination of sources.

Folists – Histosols which are saturated with water for less than 30 cumulative days during normal years (and are not artificially drained). See Keys to Soil Taxonomy (2003) for complete definition.

Folistic epipedon – A surface layer that is saturated for less than 30 days cumulatively in a normal year and consists of either organic soil material or is an Ap horizon that, when mixed to a depth of 10 inches (25 cm), has an organic carbon content of 8 percent or more if the mineral fraction contains no clay, at least 16 percent organic carbon content of the clay content is 60 percent or greater and intermediate organic carbon contents for intermediate clay contents. See Keys to Soil Taxonomy (2003).

Frequently Flooded – A frequency class in which flooding is likely to occur, often under usual weather conditions (more than 50 percent chance in any year, or more than 50 times in 100 years).

Frequently Ponded – A frequency class in which ponding is likely to occur, often under usual weather conditions (more than 50 percent chance in any year, or more than 50 times in 100 years).

Frigid Temperature Regime – A soil temperature regime that is warmer in the summer than a soil with a cryic regime, but has a mean annual soil temperature lower than 8°C. The difference between mean summer and mean winter soil temperatures is more than 6°C at a depth of 50 cm from the soil surface or at a restrictive feature in the soil. See Keys to Soil Taxonomy (2003) for complete definition.

Gleyed Matrix – A soil horizon matrix color resulting from prolonged periods of wetness that makes up more than 50% of the horizon or subhorizon where iron has been reduced and removed, or where saturation with stagnant water has preserved a reduced state.

Gley colors include:

- 10Y, 5GY, 10GY, 10G, 5BG, 10BG, 5B, 10B, or 5PB with value 4 or more and chroma is 1; or

- 5G with value 4 or more and chroma is 1 or 2; or

- N with value 4 or more.

In some places, the gleyed matrix may change color upon exposure to air. This is called a reduced matrix. This phenomenon is included in the concept of gleyed matrix in this manual.

NOTE: “g” is a horizon suffix indicating the horizon has low chroma colors due to wetness, but does not necessarily have gleyed matrix colors. All gleyed matrices should have the suffix “g”; however, all horizons that have the “g” suffix are not gleyed. For example, a horizon with the color 10YR 6/2 that is at least seasonally wet, with or without other redoximorphic features, may have the “g” suffix.

Growing Season – The portion of the year when the soil temperature is at or above biological zero at a depth of 50 cm (19.7 inches) below the soil surface (Reference: NRCS Hydric Soil Criteria as posted on http://soils.usda.gov/use/hydric/criteria.html). For purposes of this manual (specifically Category I in the “Key to Hydric Soil Indicators”), the growing season may be approximated by calculating the number of frost-free days. This information is available through local offices of the Natural Resources Conservation Service. This can include published county soil survey reports (USDA-NRCS) in the table(s) entitled “Freeze Dates in Spring and Fall.” Using this method (Row: 5 of 10 years, Column 28°F or lower), the length of the growing season would extend from the last freezing temperature in spring through the first freezing temperature in fall. (Reference: “Memorandum for Distribution”, subject: Clarification and Interpretation of the 1987 Manual, U.S. Army Corps of Engineers, Washington, D.C., March 6, 1992, Section 3.b. “Hydrology”).

Hemic Soil Material – Organic soil material with a fiber content after rubbing ranging from greater than one sixth (by volume) to less than two fifths, excluding live roots. The soil horizon notation for hemic soil material is Oe. It is intermediate decomposition between fibric and sapric.

High Value – Color values of 4 or more.

Histic Epipedon – An organic layer generally between 8 (20 cm) and 16 inches (40 cm) thick (one or more horizons) that is characterized by saturation (for 30 days or more, cumulative, in a normal year) and reduction for some time during normal years. Refer to Keys to Soil Taxonomy (2003) for complete definition.

Histosols – Organic soils that have organic soil material in more than half of the upper 80 cm (32 inches), or that are of any thickness if overlying rock or fragmental materials have interstices filled with organic soil material. When overlying unconsolidated mineral soil material, organic soils are typically greater than 16 inches thick. Refer to Keys to Soil Taxonomy (2003) for complete definition.

Horizon – See Soil Horizon.

Hydric Soil Definition (1994) – A soil that formed under conditions of saturation, flooding, or ponding long enough during the growing season to develop anaerobic conditions in the upper part. See Federal Register, July 13, 1994.

Standards for defining a functionally hydric soil are maintained by the National Technical Committee at the following website: http://soils.usda.gov/use/hydric/criteria.html.

Loamy Fine Sand or Coarser – Includes USDA Textural Classes: sands (except very fine sand) and loamy sand textures excluding loamy very fine sand.

Long Duration – A duration class in which inundation for a single event ranges from 7 to 30 days.

Low Chroma Colors – Any soil color, when using the Munsell Color System, with a chroma of 2 or less.

Many, Redoximorphic Features – Redoximorphic features comprising 20 percent or more of the observed surface area.

Masses, Soft – Redox that are not cemented and whose shapes are variable. Soft masses include features that formerly would have been called “high chroma mottles.”

Matrix Color – Dominant soil color as determined by 50 percent or more with two colors, or the highest percentage with more than two colors.

Mesic Temperature Regime – A soil temperature regime that has a mean annual soil temperature of greater than or equal to 8°C but lower than 15°C and the difference between mean summer and mean winter soil temperatures is more than 6°C at a depth of 50 cm from the soil surface or at a restrictive feature in the soil. See Keys to Soil Taxonomy (2003) for complete definition.

Mineral Soil Horizon – Soil horizon consisting of mineral soil material (e.g., A, E, and B horizons).

Mineral Soil Material – Soil comprised mostly of mineral material with a relatively low content of organic matter (less than 12 to 18 percent of organic carbon by dry weight, depending upon the clay

content). See Soil Survey Manual (1993) for complete definition. See Top of the Mineral Soil Material.

Moderate Structure – Well-formed distinct peds evident in undisturbed soil. When disturbed, soil material parts into a mixture of whole units, broken units, and material that is not in units. See Soil Survey Manual (1993) and Field Book for Describing and Sampling Soils (2002) for further description.

Mottling: –

1) A defunct term no longer used; previously used to describe

certain redoximorphic features.

2) Current usage: areas of color that differ from the matrix color or attributes retained from the geologic source rather than from pedogenesis. See Field Book for Describing and Sampling Soils (2002).

Muck – Sapric soil material that formed under wet conditions.

Mucky – A USDA soil texture modifier (e.g., mucky sand). Mucky modified mineral soil with 0 percent clay has between 5 and 12 percent organic carbon. Mucky modified mineral soil with 60 percent clay has between 12 and 18 percent organic carbon. Soils with an intermediate amount of clay have intermediate amounts of organic carbon.

Nodule (Fe/Mn) – A cemented concentration of a chemical compound of iron and manganese that can be removed from the soil intact and that has no orderly internal organization.

O Horizon – Soil horizon consisting of organic soil material.

Organic Soil Material – Soil material usually formed under wet conditions that has a minimum of 12 to 18 percent organic carbon by dry weight, depending upon the percent of clay content. See Soil Taxonomy (2001).

Organic Streaking – See stripped matrix.

Ortstein – Spodic materials that are 50% or more cemented. See Keys to Soil Taxonomy (2003).

Parent Material – The unconsolidated and more or less chemically weathered mineral or organic material from which the solum of soils has developed by pedogenic processes.

Ped – A unit of soil structure such as a block, column, granule, plate, or prism, formed by natural processes (in contrast with a clod, which is formed artificially).

Ped Interior – The interior of a soil structural unit or ped such as a block or a plate.

Pedon – A three-dimensional body of soil with lateral dimensions large enough to permit the study of horizon shapes and relations. See Soil Survey Manual (1993) for a complete definition.

Ponded – A condition in which water stands in a closed depression. The water is removed only by percolation, evaporation and/or transpiration.

Pore Linings – Zones of accumulation of Fe and/or Mn that occur as either coatings on a pore surface or impregnations of the matrix adjacent to the pore. See Vepraskas (1996) or Richardson and Vepraskas (2001) for complete discussion. Some pore linings are strongly expressed oxidized rhizospheres.

Profile – A vertical exposure of the soil.

Prominent, Redoximorphic Feature – A term used to describe the contrast in color between a redoximorphic feature and the matrix color. Refer to both the chart and tabular key for estimating contrast of redoximorphic features, located in Appendices 4 and 5.

Redox Concentrations – Features formed when Fe [or Mn] oxides or hydroxides have accumulated at a point or around a large pore such as a root channel. They appear to have formed by Fe or Mn moving into an area, oxidizing, and precipitating. Redox concentrations contain more Fe3+ [or Mn4+] than were found in the soil matrix originally. Three groups of redox concentrations are recognized: 1) Fe [or Mn] masses, 2) pore linings, and 3) nodules and concretions. See Richardson and Vepraskas (2001)

Redox Depletions – Zones of low chroma (2 or less) having value 4 or more where Fe-Mn oxides have been stripped or where both

Fe-Mn oxides and clay have been stripped. Redox depletions include features that would formerly have been called “gray mottles.” See Richardson and Vepraskas (2001) for more complete discussion.

Redoximorphic Features – Features associated with wetness formed by the processes of reduction, translocation, and/or oxidation of Fe and Mn oxides. Formerly called mottling and low chroma colors. Redoximorphic features include: soft masses, pore linings, nodules and concretions, iron depletions, clay depletions, depleted matrices, gleyed matrices,and reduced matrices. See Vepraskas (1996) for more complete discussion.

Redoximorphic Features, Abundance of – See Abundance, Redoximorphic Features.

Redoximorphic Features, Contrast of – See Contrast, Redoximorphic Features.

Redoximorphic Features, Size of – See Size, Redoximorphic Features.

Reduced Matrix – Soil matrices that have low chroma and high value colors, but whose color changes in hue or chroma when exposed to air. See gleyed matrix. See Richardson and Vepraskas (2001) for complete discussion.

Restrictive feature – A root-limiting layer in the soil profile such as a lithic contact (bedrock) or densic materials. See Keys to Soil Taxonomy (2003) for complete definition.

Sapric Soil Material – Organic soil material that has a fiber content after rubbing of less than one sixth (by volume), excluding live roots. The soil horizon notation for sapric soil material is Oa.

Shallow E Horizon – An E horizon which does not extend below a depth of 6 inches. Measurement is made from the top of the mineral soil material.

Size, Redoximorphic Features – The size of redoximorphic features (fine, medium, coarse, very coarse, and extremely coarse). Refer to the Chart for Estimating Size of Redoximorphic Features, located in Appendix 2.

Slightly Decomposed Organic Soil Material – Refer to fibric soil material.

Soil – Soil is a natural body comprised of mineral and organic matter, liquid, and gases that occurs on the land surface. Soils are characterized by one or both of the following: 1) horizons, or layers, that are distinguishable from the initial material as a result of additions, losses, transfers, and transformations of energy and matter or 2) the ability to support rooted plants in a natural environment.

Soil Horizon – A relatively uniform layer of soil or soil material approximately parallel to the land surface and differing from adjacent genetically related layers in physical, chemical, and biological properties or characteristics such as color, structure, texture, consistence, degree of acidity, and presence or absence of redoximorphic features, etc.

Soil Morphology – The physical constitution of a soil profile as exhibited by the kinds, thicknesses, and arrangements of the horizons in the profile, and by the texture, structure, consistence, porosity and color of each horizon.

Soil Structure – The combination or arrangement of primary soil particles into secondary units or peds. The secondary units are characterized on the basis of size, shape, and grade (degree of distinctness). See Appendix 5.

Soil Surface – The top of the uppermost soil horizon (ground surface). Fresh leaf or needle fall that has not undergone observable decomposition is excluded from soil and may be described separately.

Solum – A set of horizons that are related through the same cycle of pedogenic processes; the solum may include the O, A, E,and B horizons.

Spodic Horizon – An illuvial layer that is 1 in. (2.5 cm) or more thick, is not part of an Ap horizon, and contains 85 percent or more spodic materials. For field conditions where an albic (E) horizon extends horizontally through 50 percent or more of each pedon, the moist, crushed and smoothed colors directly under the albic horizon are as follows:

- a hue 5YR or redder (i.e., 2.5YR, etc.); or

- a hue of 7.5YR, color value 5 or less and chroma 4 or less (a hue of 7.5YR with value 5 and chroma 6 does not qualify without additional chemical properties); or

- a hue of 10YR or neutral and color value and chroma 2 or less; or

- a color of 10YR 3/1.

See Keys to Soil Taxonomy (2003) for complete definition.

Spodic Materials – Mineral soil materials that are dominated by illuvial active amorphous materials composed of organic matter and aluminum with or without iron. Color criteria are given in the definition of spodic horizon above. See Keys to Soil Taxonomy (2003) for complete definition.

Stripped Matrix – A layer with features associated with wetness in which iron/manganese oxides and/or organic matter have been stripped, exposing the primary base color of the soil materials. The stripped areas and translocated oxides and/or organic matter form a diffuse splotchy pattern of two or more colors. Commonly the splotches of color are shades of gray or grayish brown. See Field Indicators (2002).

Strong Structure – Peds are distinct in undisturbed soil. They separate cleanly when soil is disturbed, and the soil material separates mainly into whole units when removed.

Sulfidic Materials – Mineral or organic soil material that contains oxidizable sulfur compounds. Sulfidic materials most commonly accumulate in coastal marshes, but they may occur in freshwater marshes if there is sulfur in the water. Presence of hydrogen sulfide gas is a field indicator for identifying sulfidic materials. It is detected in the field by a rotten egg smell. See Keys to Soil Taxonomy (2003) for a complete definition.

Thick, A and Ap Horizons – For the purposes of this manual, thick means greater than 10 inches and less than or equal to 15 inches.

Top of the Mineral Soil Material – Typically the top of the uppermost mineral soil horizon. An exception may be an area of organic soil material buried beneath mineral soil material. See Keys to Soil Taxonomy (2003) for complete discussion. This was referred to as the “mineral soil surface” in previous versions of this key.

Upper Part of the Soil – That portion of the soil that supports the majority of plant roots. In general, in wetlands this zone is limited to the upper 12 inches (30 cm).

Very Dark A or Ap Horizon – Refer to A or Ap Horizon, Very Dark.

Very Long Duration – This is a duration class in which inundation for a single event is greater than 30 days.

Very Thick, A and Ap Horizons – For the purposes of this manual, very thick means greater than 15 inches (38 cm).

Well-Decomposed Organic Soil Material – Refers to sapric soil material.

References

Childs, C.W. 1981. Field test for ferrous iron and ferric-organic complexes (on exchange sites or in water-soluble forms) in soils. Aust. J. Soil. Res. 19:175-180.

Federal Register. July 13, 1994. Changes in Hydric Soils of the United States. Washington, DC (current Hydric Soil Definition).

Glossary of Soil Science Terms. 1997. Soil Science Society of America, Madison, WI.

Richardson, J.L. and M.J. Vepraskas (editors). 2001. Wetland Soils: Genesis, Hydrology, Landscapes, and Classification. Lewis Publishers CRC Press LLC, 2000 N.W. Corporate Blvd., Boca Raton, Florida.

Department of Agriculture, Natural Resources Conservation Service. 2002. Field Book for Describing and Sampling Soils, Version 2.0. P.J. Schoeneberger, D.A. Wysocki, E.C. Benham, and W.D. Broderson (eds.). National Soil Survey Center, Lincoln, NE.

Department of Agriculture, Natural Resources Conservation Service. 2002. Field Indicators of Hydric Soils in the United States, Version 5.0. G.W. Hurt, P.M. Whited, and R.F. Pringle (eds.). USDA-NRCS in cooperation with the National Technical Committee for Hydric Soils, Fort Worth, TX.

Department of Agriculture, Natural Resources Conservation Service. 2003. New Edition of the Keys to Soil Taxonomy, Ninth Edition. C. Ditzler, National Soil Survey Center, Lincoln, NE.

Department of Agriculture, Soil Survey Division Staff, 1993. Soil Survey Manual, Handbook No. 18. US Government Printing Office, Washington, DC.

Department of Agriculture, Soil Survey Division Staff. 1999. Soil Taxonomy, Second Edition. Handbook Number 436. US Government Printing Office, Washington, DC.

Vepraskas, M.J. 1996. Redoximorphic Features for Identifying Aquic Conditions. Technical Bulletin 301. North Carolina Agricultural Research Service, North Carolina State University, Raleigh, NC.